# Coating Sequence State Machine — Failure Analysis

Source files: Subroutines1.c, Subroutines.c
State machine location: doevents()  —  Subroutines1.c:1202
State definitions: Subroutines.h:331-354

# State Flow Overview

TrigState (1)

└─ [LA still moving > 5s]      ──► ErrorState (99)

└─ [LA not moving]             ──► CkHeadRotation (2)

CkHeadRotation (2)

└─ [StateTime > 5s]            ──► HeadErrorState (100)

└─ [HeadSpeed > 5000 RPM]      ──► PurgeRetract (3)

PurgeRetract (3)

└─ [after 2s, purge retracted] ──► InitLAMove (5)

└─ [after 2s, still moving]    ──► PurgeRetractWait (4)

PurgeRetractWait (4)

└─ [StateTime > 20s]           ──► ErrorState (99)

└─ [purge retracted]           ──► InitLAMove (5)

InitLAMove (5)

└─ [LAPos != MaxLADist]        ──► HomeState (12)   [recovery]

└─ [no strokes configured]     ──► ErrorState (99)  [config error]

└─ [FirstStrokes > 0]          ──► StartCoatState (6)

└─ [SecondStrokes > 0]         ──► StartSecondCoatState (8)

└─ [CleanOutStrokes > 0]       ──► StartCleanOutState (10)

StartCoatState (6)

└─ [!LAMovingTimer — BUG]      ──► ErrorState (99)

└─ [LA not moving]             ──► FirstCoatState (7)

FirstCoatState (7)

└─ [LAError]                   ──► ErrorState (99)

└─ [HeadSpeed < 5000]          ──► HeadErrorState (100)

└─ [FirstCoatSeq done]         ──► StartSecondCoatState (8)

StartSecondCoatState (8)

└─ [LA not moving]             ──► SecondCoatState (9) or StartCleanOutState (10)

SecondCoatState (9)

└─ [LAError]                   ──► ErrorState (99)

└─ [HeadSpeed < 5000]          ──► HeadErrorState (100)

└─ [SecondCoatSeq done]        ──► StartCleanOutState (10)

StartCleanOutState (10)

└─ [LA not moving]             ──► CleanOutState (11)

CleanOutState (11)

└─ [LAError]                   ──► ErrorState (99)

└─ [CleanCoatSeq done]         ──► HomeState (12)

HomeState (12)

└─ [LA not moving]             ──► StopState (13)  [both branches — see §3.1]

StopState (13)

└─ [!LAMovingTimer — BUG]      ──► ErrorState (99)

└─ [LA not moving]             ──► CoatingComplete (14)

CoatingComplete (14)

└─ [after 1s]                  ──► StopWaitState (15)

StopWaitState (15)

└─ [Cycle_Complete — always 1] ──► FinishState (16)

└─ [StateTime > 15s — dead]    ──► default → FinishState (16)

FinishState (16)  ──  stays here until next trigger

ErrorState (99)   ──  turns off head + pump  ──► FinishState (16)

HeadErrorState (100)  ──  turns off head + pump  ──► HdErrHomeState (101)

HdErrHomeState (101)

└─ [LA not moving]             ──► HdErrStopState (102)  [both branches — see §3.2]

HdErrStopState (102)

└─ [LA not moving]             ──► default → FinishState (16)

# Section 1 — Transitions to ErrorState (99):  "Warn:TIMEOUT ERROR"

## 1.1  Actuator Still Moving at Trigger — TrigState  (line 1226)

else if ( StateTime > RTI_One_Sec * 5 )

State = ErrorState;

If the LA is still moving when the PLC trigger fires, it gets 5 seconds to finish. If it does not stop within that window, the sequence errors before it begins.

## 1.2  No Stroke Sequences Configured — InitLAMove  (line 1281)

else

State = ErrorState;

If FirstStrokes, SecondStrokes, and CleanOutStrokes are all zero, the state machine has nowhere to go and immediately transitions to ErrorState. This is a configuration error.

## 1.3  LAMovingTimer Check Outside the Guard Block — StartCoatState  (line 1303)

if ( !gProcImg[OUT_digi_7] )  // guard block

{

FirstCoatSeq( 1 );        // sets LAMovingTimer inside

State++;

}

if (!LAMovingTimer)           // runs unconditionally, even while LA is moving

State = ErrorState;

This check runs on every pass through StartCoatState, not just when the LA is idle. If LAMovingTimer happened to expire before this state was entered (e.g., the LA was slow to stop from a prior move), the sequence errors out immediately with no display message, before any coating has started.

This is the most common source of a phantom timeout that appears unrelated to coating.

## 1.4  Purge Unit Did Not Retract — PurgeRetractWait  (line 1256)

if ( StateTime > RTI_One_Sec * 20 )

State = ErrorState;

20-second timeout waiting for IN_digi_31 to go low. A physical jam or CAN bus failure causes ErrorState.

## 1.5  LAMovingTimer Check Outside the Guard Block — StopState  (line 1411)

if ( !gProcImg[OUT_digi_7] )

{

MoveLA ( MaxLADist.value, 100, LACurrent );  // sets LAMovingTimer

State = CoatingComplete;

}

if (!LAMovingTimer)                              // unconditional -- same bug as 1.3

State = ErrorState;

Identical structural bug to StartCoatState. If LAMovingTimer expired before entering this state, the sequence errors instead of extending to max and completing.

## 1.6  LAError Set in Coating Sequence Functions

LAError = 1 is raised inside FirstCoatSeq, SecondCoatSeq, and CleanCoatSeq. FirstCoatState, SecondCoatState, and CleanOutState each check it and transition to ErrorState. Two conditions set LAError:

LA did not start moving within 1 second (LAMoveTimer = 1s, set in MoveLA()):

else if ( MoveCmdXmtd )

{

if (!LAMoveTimer)

{

LAError = 1;

Display ("Proc:Not Moving");

}

return 0;

}

LA kept moving past its computed timeout (LAMovingTimer based on distance + speed):

if (!Temp_LAMovingTimer)

{

LAError = 1;

Display ("Proc:Moving Too Long");

}

The sequence functions use interrupt-protected reads of LAMovingTimer via Temp_LAMovingTimer. The unprotected reads in StartCoatState and StopState (1.3, 1.5) are the risky ones.

# Section 2 — Transitions to HeadErrorState (100):  "Warn:HEAD ERROR"

## 2.1  Head Did Not Reach Speed — CkHeadRotation  (line 1233)

if ( StateTime > RTI_One_Sec * 5 )

State = HeadErrorState;     // evaluated first

if ( HeadSpeed > 5000 )

State++;

HeadSpeed is reset to 0 in TrigState and then updated from tach input captures in Interrupts.c. The 5-second timeout is checked before the speed check in execution order, so if both conditions become true on the same tick, HeadErrorState wins even if the head is actually spinning.

## 2.2  Head Slowed During Coating — FirstCoatState / SecondCoatState  (lines 1311, 1338)

if ( HeadSpeed < 5000 )

State = HeadErrorState;

HeadSpeed is set to 0 in Interrupts.c when the tach input times out. Any loss of the tach signal (dirty sensor, brief disconnection, or ECU reset) sets HeadSpeed = 0 and immediately fires HeadErrorState mid-coat.

Note: CleanOutState does not have this check. The head can stop during cleanout without triggering an error.

## 2.3  Head Error Recovery Path

HeadErrorState turns off the head and pump, then advances to HdErrHomeState -> HdErrStopState, which extends the LA to max before returning to FinishState. See Section 3.1 and 3.2 for bugs in this recovery path.

# Section 3 — Logic Bugs That Cause Incorrect Behavior

## 3.1  HomeState: Both Branches Lead to the Same State  (line 1388)

case HomeState:

if ( !gProcImg[OUT_digi_7] )

{

if (gProcImg[IN_digi_12] != 0)

State++;           // 12 + 1 = 13 = StopState

else

State = StopState; // also 13

}

The position check is meaningless. State++ from HomeState (12) equals StopState (13), the same value as the explicit assignment. The branch intended to send the LA home first does nothing different. Both paths immediately fall into StopState, which extends the LA to max.

Additionally, IN_digi_12 is read regardless of LA_TYPE. For LA_TYPE == 1 (Cleaner), position lives at IN_digi_32. Since both branches lead to the same state, this does not change the outcome here, but is consistent with the wrong-register issue in 3.2.

## 3.2  HdErrStopState: Unit Mismatch on Position Check  (line 1490)

if (gProcImg[IN_digi_12] != MaxLADist.value)

State++;                  // 102 + 1 = 103 -> default -> FinishState

else

State = FinishState;      // dead branch

IN_digi_12 stores Pos x 10 (set by MoveLA(): gProcImg[IN_digi_12] = Pos*10), so for a 12-inch machine it holds 120. MaxLADist.value is in whole inches (12). The comparison 120 != 12 is always true, so the State = FinishState branch is unreachable. Both paths lead to FinishState through different routes, so the final outcome is the same, but the intended branch is never taken.

## 3.3  CleanCoatSeq: Array Out-of-Bounds When Both Stroke Counts Are Zero  (line 462)

else  // SecondStrokes == 0

{

Len = FirstStrokeLen[(int)FirstStrokes.value - 1].value;  // index = -1 !

...

}

If SecondStrokes = 0 and FirstStrokes = 0, the array index is -1. This produces undefined behavior and reads memory before the start of the array. The only way to reach CleanOutState with both counts at zero is if CleanOutStrokes > 0 while the other two are zero, a configuration that InitLAMove allows through (Section 1.2 only errors when all three are zero).

## 3.4  Operator Precedence Bug in MoveLA()  (lines 267, 285)

// As written:

if ( gProcImg[IN_digi_12+2] & ~0x01 != Current & ~0x01 )

// How C evaluates it (& has lower precedence than !=):

if ( gProcImg[IN_digi_12+2] & (~0x01 != Current) & ~0x01 )

~0x01 != Current evaluates to 0 or 1 before the & operators run. ANDed with ~0x01 = 0xFE, the result is always 0. The current-limit byte check never fires. The intended expression was:

if ( (gProcImg[IN_digi_12+2] & ~0x01) != (Current & ~0x01) )

The current limit byte is written unconditionally without validation every time MoveLA() is called.

## 3.5  StopWaitState: Timeout and Cycle_Complete Both Effectively Dead  (line 1426)

// Three lines before the switch statement:

if ( 1 )

Cycle_Complete = 1;

// Inside the switch:

case StopWaitState:

if ( StateTime > 15 * RTI_One_Sec )

State = 99;           // hits default -> FinishState (not ErrorState)

if ( Cycle_Complete )

State++;              // always fires immediately

Cycle_Complete is set to 1 unconditionally before the switch every time doevents() runs (except when TrigState resets it to 0). By the time execution reaches StopWaitState, Cycle_Complete is always 1, so the state always advances on the first tick. The 15-second timeout never fires. Additionally, State = 99 hits the default: case which resolves to FinishState, not ErrorState, so it would not produce a timeout warning even if it did fire.

## 3.6  update_menu_var_by_value() Silently Fails on Out-of-Range Values  (line 2037)

if (new_value < var->min || new_value > var->max)

return -1;  // silent failure -- no caller checks this

Called at the end of FirstCoatSeq and SecondCoatSeq to restore PumpSpd to OldPumpSpeed. If OldPumpSpeed was captured with a value outside the menu variable's configured range, the pump speed is silently left at whatever it was during the last stroke rather than being restored. No caller checks the return value.

# Section 4 — Summary Table

| # | State | Failure / Bug | Outcome | Display |
| --- | --- | --- | --- | --- |
| 1.1 | TrigState | LA still moving at trigger, > 5s timeout | ErrorState | "Warn:TIMEOUT ERROR" |
| 1.2 | InitLAMove | All stroke counts are zero (config error) | ErrorState | "Warn:TIMEOUT ERROR" |
| 1.3 | StartCoatState | !LAMovingTimer check outside if(!moving) guard — phantom timeout | ErrorState | "Warn:TIMEOUT ERROR" |
| 1.4 | PurgeRetractWait | Purge unit fails to retract within 20s | ErrorState | "Warn:TIMEOUT ERROR" |
| 1.5 | StopState | Same unconditional !LAMovingTimer bug as 1.3 | ErrorState | "Warn:TIMEOUT ERROR" |
| 1.6a | First/Second/CleanCoatSeq | LA doesn't begin moving within 1s of command | ErrorState (next tick) | "Proc:Not Moving" |
| 1.6b | First/Second/CleanCoatSeq | LA exceeds computed travel time | ErrorState (next tick) | "Proc:Moving Too Long" |
| 2.1 | CkHeadRotation | Head doesn't exceed 5000 RPM within 5s | HeadErrorState | "Warn:HEAD ERROR" |
| 2.2 | FirstCoatState / SecondCoatState | Head drops below 5000 RPM mid-coat | HeadErrorState | "Warn:HEAD ERROR" |
| 3.1 | HomeState | Both position branches lead to same state — position check is dead code | Wrong branch label; same outcome | — |
| 3.2 | HdErrStopState | IN_digi_12 (Pos x 10) compared against MaxLADist (whole inches) — always unequal | Correct branch never taken; same final outcome | — |
| 3.3 | CleanCoatSeq | Array index FirstStrokeLen[-1] when FirstStrokes=0 and SecondStrokes=0 | Undefined behavior / bad memory read | — |
| 3.4 | MoveLA() | Operator precedence bug — current-limit check always evaluates false | Current byte written without validation | — |
| 3.5 | StopWaitState | Cycle_Complete always 1; 15s timeout resolves to FinishState not ErrorState | Both paths are dead code | — |
| 3.6 | First/SecondCoatSeq | update_menu_var_by_value() silently rejects out-of-range OldPumpSpeed | Pump speed not restored after coating | — |